SPEC DATA SHEET # GWC_EX-N

4700 Clairton Blvd., Pittsburgh, PA 15236

Tel: (412) 884-3000 • Fax: (412) 884-3300

www.oeler.com	•	sales@oeler.com

Intelligent Acoustic Solutions

# GWC_EX-N

# Sound Curtains

Oelex GWC-EX-N by Oeler Industries, Inc. is	an	economical			noise		barrier	for outdoor			applications.			This		barrier-wall product		consist		of			a				one-pound		 per sq. ft. non-reinforced loaded vinyl noise barrier.				Modular				curtain		 panels		are constructed with grommets across the top and bottom, and exterior				grade			Velcro seals	 along	the				vertical		edges. Modular					panels			are			sewn		with			an exterior grade thread.

# FEATURES:

- STC 26 Rating

- Economical short-term solution

- Available barrier colors: black

4700 Clairton Blvd., Pittsburgh, PA 15236

Tel: (412) 884-3000 • Fax: (412) 884.3300

www.oeler.com	•	sales@oeler.com

Intelligent Acoustic Solutions

Intelligent Acoustic Solutions

SPEC DATA SHEET # GWC_EX-N

APPLICATIONS:

Typically used as modular curtain panels on short-term construction projects or other applications such as noise control around drilling equipment, enclosing HVAC equipment, dust collectors or similar machinery behind a manufacturing plant.

This is an economical solution where cost is an issue during your project. Also available with a two-inch thick quilted fiberglass absorber, or with a two-pound per sq. ft. reinforced barrier.

## PRODUCT DATA

Description: 1 lb-psf non-reinforced loaded vinyl noise barrier

Nominal Thickness: 0.107 inch

Standard Panel Width: 54” wide, lengths as required up to 16’ high

Weight: 1.0 lb psf

## ACOUSTICAL PERFORMANCE:

Sound Transmission Loss:

OCTAVE BAND FREQUENCIES (HZ)

| Product | 125 | 250 | 500 | 1K | 2K | 4K | NRC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GWC-EX-N | 13 | 17 | 22 | 26 | 32 | 37 | 26 |

ASTM E-90 & E 413