SPEC DATA SHEET # SONIC PRINTING

4700 Clairton Blvd., Pittsburgh, PA 15236

Tel: (412) 884-3000

www.oeler.com

•

•

Fax: (412) 884-3300

sales@oeler.com

Intelligent Acoustic Solutions

SONIC PRINTING

PRINTED ACOUSTIC PANELS

Oelex Custom Printed Acoustical Wall Panels

by

Oeler

Industries,

Inc.	offer

a

visually

appealing

problems.

way  to  resolve  reverberant	noise

Add  your  logo  or  custom  art  on

these	premium	acoustic	panels	guaranteed

to

improve

the

aesthetics

and

acoustics

of your existing space.

APLICATIONS:

•

•

•

•

•

•

•

Restaurants

Office spaces Schools Universities Airports Churches

Any area where excess

reverberation is a problem

FEATURES:

•

•

•

•

Custom sizes available

Excellent acoustical values

Visually appealing acoustic solution Custom printed

Intelligent Acoustic Solutions

SPEC DATA SHEET # PRINTED ACOUSTIC PANELS

SIZING:

Available in standard and custom sizes up to a maximum of 4’x10‘.

THICKNESS:

- Standard thicknesses: 1” and 2”

- Custom thicknesses available

SUBSTRATES:

6-7# PCF rigid fiberglass core

EDGE DETAIL:

A chemically hardened edge comes standard on all panels unless otherwise requested.

- Square

- Radius

- Bevel

- Miter

MOUNTING OPTIONS:

- Rotofast Snap-on Anchors

- Impaling Clips

- 2-Part Mechanical Clips

ACOUSTICAL PERFORMANCE:

1/3 Octave Sound Absorption Coefficients at the Octave Band Frequencies

4700 Clairton Blvd., Pittsburgh, PA 15236

Tel: (412) 884-3000 • Fax: (412) 884.3300

www.oeler.com

•

sales@oeler.com

Intelligent Acoustic Solutions

Frequency (Hz)	125	250	500	1K	2K	4K	NRC	SAA

1” Thick	0.04	0.29	0.74	1.00	1.01	1.00	0.75	0.77